BLE File Transfer SOP

Version 1.0

Author: Your Name

Date: June 6th, 2026

Script Documentation: BLE-Based SD Card File Transfer Manager

Script Overview

This script implements a Bluetooth Low Energy (BLE) file transfer manager that allows a remote device (such as a phone or computer) to securely request and receive files stored on an SD card.

The system uses a UART-style BLE service to receive commands, authenticate file requests, and transmit file contents in small chunks in a non-blocking and power-efficient manner.

Purpose & Design Goals

This script was designed to:

- Enable wireless file retrieval from an SD card

- Avoid blocking the main system loop

- Minimize power consumption during BLE operation

- Protect files using command-based authorization

- Safely coordinate SD card access with other system components

It is especially suitable for embedded and battery-powered devices.

Hardware & Communication

- BLE Interface: MicroPython ubluetooth

- Storage: SD card (SPI-based)

- Transport Protocol: BLE UART-style service

- Chunk Size: Default 20 bytes (BLE MTU safe)

The script assumes the SD card and BLE stack are already supported by the hardware platform.

BLE UART Service Definition

The script defines a Nordic UART–compatible BLE service:

| Component | UUID | Purpose |
| --- | --- | --- |
| UART Service | 6E400001-... | Main BLE service |
| TX Characteristic | 6E400003-... | Notifications to client |
| RX Characteristic | 6E400002-... | Commands from client |

This allows easy integration with common BLE terminal apps.

BLEManager Class

Purpose

Encapsulates BLE setup, command handling, SD card access, and file transfer logic.

Initialization (__init__)

During initialization, the manager:

- Activates the BLE radio

- Stores SD card and SPI references

- Loads authorized file mappings

- Sets transfer parameters (chunk size, BLE name)

- Registers BLE services

- Begins advertising automatically

SD Card Mounting

Function: mount_sd()

Purpose

Mounts the SD card filesystem safely and idempotently.

Key Behaviors

- Prevents duplicate mounts

- Handles OS-level mounting errors gracefully

- Mounts the SD card at /sd

This function allows BLE operations to recover cleanly if the SD card is temporarily unavailable.

BLE Advertising

Function: advertise()

Behavior

- Advertises the device with a configurable name

- Truncates long names to reduce BLE payload size

- Uses a 500 ms advertising interval to reduce power usage

Advertising automatically restarts after a client disconnects.

BLE Event Handling

Function: _bt_irq(event, data)

Handles core BLE events:

| Event | Action |
| --- | --- |
| Central connect | Store connection handle, stop advertising |
| Central disconnect | Cleanup state, resume advertising |
| GATT write | Process incoming command |

This keeps BLE logic centralized and efficient.

Command Handling

Function: _handle_command()

Supported Commands

- Authorized file keys → begin file transfer

- STOP → abort active transfer

- Invalid commands → rejected with response

Commands are matched against a predefined file map, preventing unauthorized file access.

File Transfer Logic

Non-Blocking Design

File transfer is handled incrementally through the update() method, which:

- Sends one line per call

- Avoids blocking loops

- Coordinates SD card access via a shared busy flag

Transfer Flow

- Client sends authorized command

- File is opened

- TRANSFER_START is sent

- Each line is sent with an index prefix

- TRANSFER_COMPLETE is sent at EOF

- Resources are cleaned up

Data Transmission

Function: send_line()

Key Features

- Automatically splits data into BLE-safe chunks

- Respects MTU limits

- Adds pacing delays for large transfers

- Handles lost connections gracefully

This ensures reliable data delivery across a constrained BLE channel.

Logging Support

Function: log(message)

- Prints messages locally

- Sends log output over BLE if connected

This allows real-time monitoring during remote operation.

Resource Cleanup

Functions

- _cleanup_transfer() – resets file and transfer state

- deinit() – shuts down BLE, SD usage, and advertising

These ensure no resources remain locked after disconnects or shutdowns.

Outputs

- BLE status messages

- File contents transmitted line-by-line

- Transfer state messages (START, COMPLETE, ERROR)

- Optional BLE log messages

Dependencies

- machine (SPI, GPIO)

- ubluetooth

- sdcard

- os

- struct

- time

- SD_Card (shared SD state)

- log_manager (optional logging integration)

Design Notes & Limitations

- BLE transfer speed is intentionally limited for reliability

- File access is read-only

- Authentication is command-based, not encrypted

- SD card must not be busy with other operations

- Client must re-request transfer after disconnect